SmartTwin

AI-Enabled Digital Twin for Predictive Maintenance

ESP32  ·  XGBoost  ·  Python Flask  ·  React + Vite  ·  HTTP / Local Network  ·  Industry 4.0

# Overview

SmartTwin is an end-to-end Industry 4.0 / IIoT project demonstrating a complete predictive maintenance pipeline. The ESP32 reads vibration, temperature, current, humidity, and RPM from a running motor and sends the data over HTTP to a Python Flask backend on the same local network. The backend runs an XGBoost ML model and streams results to the React frontend in real time.

# System Architecture

Physical Motor  →  ESP32 Sensors  →  HTTP (Local Network)  →  Python Backend  →  React Dashboard

↓

XGBoost ML Model

(Fault Classification)

Note: Both the ESP32 and the computer running the backend must be on the same Wi-Fi network.

# Features

- Real-time sensor streaming over HTTP from ESP32 to backend

- 6-class fault classification using a pre-trained XGBoost model (no retraining required to run)

- Vibration severity levels: NONE / LOW / MEDIUM / HIGH / CRITICAL

- Live React dashboard showing RPM, temperature, current, humidity, and fault state

- Predictive alerts — flags anomalies before equipment fails

- Retrain support via python retrain.py using your own data

- Full model comparison across 6 ML algorithms (notebooks included)

# Hardware & Sensors

| Sensor | Parameter Measured | ESP32 Interface |
| --- | --- | --- |
| MPU6050 | Vibration (Accel/Gyro) | I²C — SDA: GPIO23, SCL: GPIO22 |
| LM35 | Motor Temperature (°C) | Analog — GPIO34 |
| DHT22 | Ambient Temp & Humidity | Digital — GPIO4 |
| ACS712 | Motor Current (A) | Analog — GPIO35 |
| Rotary Encoder | RPM | Digital — GPIO18/19 |

Note: Clone MPU6050 modules (WHO_AM_I = 0x70) are supported via a firmware workaround. SDA is mapped to GPIO23 (not the default GPIO21) to avoid I²C conflicts.

CSV output format from ESP32:

RPM, Current (A), Temperature (°C), Humidity (%), Vibration (g), Vibration_Level

# Fault Classification

The ML model classifies the motor's operating state into 6 classes:

| Class | Description |
| --- | --- |
| Normal | Motor running within healthy parameters |
| Overload | Current draw exceeds rated threshold |
| Overheating | Motor temperature above safe limits |
| Mechanical_Vibration | Excessive vibration indicating bearing/alignment issues |
| Critical_Fault | Multiple parameters simultaneously out of range |
| Stopped_Idle | Motor at rest / not running |

# ML Pipeline

The pre-trained model ships with the repo — you do not need to retrain it to run the project.

### Training Notebooks (Reference)

| Notebook | Purpose |
| --- | --- |
| Model2.ipynb | XGBoost training on Dataset.csv |
| Model Comparison.ipynb | Benchmark of 6 classifiers |
| test.ipynb | Validation and testing |

### Retraining

To retrain the model with new data:

cd Final

python retrain.py

### Model Comparison Results

| Model | Accuracy | Precision | Recall | F1 |
| --- | --- | --- | --- | --- |
| XGBoost ⭐ | ~92% | Best | Good | Good |
| Random Forest | ~91% | Good | Best | Best |
| SVM | ~88% | Good | Moderate | Moderate |
| Decision Tree | ~85% | Moderate | Moderate | Moderate |
| KNN | ~84% | Moderate | Moderate | Moderate |
| Logistic Regression | ~78% | Lower | Lower | Lower |

Note: XGBoost is selected as the production model for its high accuracy and precision across all fault classes.

# Repository Structure

Smart-Twin/

│

├── Arduino_Code/               # Prototype ESP32 firmware

│

├── Final Arduino Code/         # Production ESP32 firmware

│   └── *.ino                   # Sensor integration + HTTP POST to backend

│

├── Final/                      # Main application

│   ├── app.py                  # Python Flask backend — receives data, runs ML inference

│   ├── retrain.py              # Script to retrain the XGBoost model

│   ├── frontend/               # React + Vite frontend

│   │   ├── src/

│   │   │   └── components/     # ← Put your backend IP address here

│   │   └── package.json

│   └── ...

│

├── Dataset.csv                 # 4,000-row motor fault dataset

├── Model2.ipynb                # XGBoost training notebook

├── Model Comparison.ipynb      # 6-model comparison notebook

├── model_comparison_results.csv

└── test.ipynb

# Getting Started

## Prerequisites

- Hardware: ESP32, MPU6050, LM35, DHT22, ACS712, Rotary Encoder

- Software: Arduino IDE, Python >= 3.8, Node.js >= 16

- Network: ESP32 and your computer on the same Wi-Fi network

## Step 1 — Configure & Flash the ESP32

- Open Final Arduino Code/ in Arduino IDE

- Install required libraries: Adafruit_MPU6050, DHT sensor library, ArduinoJson, HTTPClient

- In the sketch, set your Wi-Fi credentials:

const char* ssid     = "YOUR_WIFI_SSID";

const char* password = "YOUR_WIFI_PASSWORD";

- Set the backend IP (your computer's local IP, e.g. 192.168.1.10):

const char* serverURL = "http://192.168.1.10:5000/data";

- Flash to the ESP32

## Step 2 — Start the Backend

cd Final

# Install Python dependencies

pip install -r requirements.txt

# Start the Flask backend

python app.py

The backend listens on port 5000 and runs ML inference on every incoming data point.

## Step 3 — Configure & Start the Frontend

- Open Final/frontend/src/components/ and update the backend IP to match your machine:

const API_URL = "http://192.168.1.10:5000";

- Install dependencies and run the dev server:

cd Final/frontend

npm install

npm run dev

- Open the URL shown in the terminal (typically http://localhost:5173) in your browser.

## Step 4 — Power the ESP32

Once the ESP32 is powered and connected to Wi-Fi, it will start sending sensor readings to the backend automatically, and the dashboard will update in real time.

# Dataset

Dataset.csv contains 4,000 rows with the following columns:

| Column | Unit | Description |
| --- | --- | --- |
| RPM | rev/min | Motor speed |
| Current | A | Phase current |
| Temperature | °C | Motor winding temperature |
| Humidity | % | Ambient relative humidity |
| Vibration | g | RMS vibration magnitude |
| Fault_Label | — | Target class (6 categories) |

Synthetically generated with Gaussian noise per class, realistic operating envelopes per fault type, and balanced class distribution.

# Model Performance

XGBoost final results on the test split:

| Metric | Score |
| --- | --- |
| Accuracy | ~92% |
| Macro Precision | ~91% |
| Macro Recall | ~90% |
| Macro F1 | ~90% |

# Tech Stack

| Layer | Technology |
| --- | --- |
| Firmware | C++ (Arduino) |
| Communication | HTTP over local Wi-Fi |
| Backend | Python · Flask · XGBoost · scikit-learn |
| Frontend | React · Vite · JavaScript · CSS |
| ML Notebooks | Jupyter · Pandas · scikit-learn |

Built as part of an Industry 4.0 / IIoT research initiative.